CircuitMind

AI-Powered Electronics Prototyping Tool

Hackathon Design Document  •  v1.0

May 2026

# 1. Project Overview

CircuitMind is an AI-powered electronics prototyping tool designed for robotics teams and hardware engineers. It eliminates the need for heavyweight desktop CAD tools like SolidWorks by providing a browser-based, conversational interface to design, simulate, and iterate on circuits — on any platform including Mac.

The core loop is:

- Upload a component datasheet or describe a circuit idea in plain English

- An AI agent parses the intent and generates a structured circuit definition

- The circuit renders as an interactive breadboard visualization in the browser

- The user chats with the agent to iterate on the design, step-by-step

- Current flow is animated to help users understand how the circuit behaves

# 2. Problem Statement

| Pain Point | Description |
| --- | --- |
| Tool accessibility | SolidWorks and similar tools are Windows-only, expensive, and overkill for quick prototypes. |
| Learning curve | Junior engineers spend hours learning CAD before they can prototype a simple circuit. |
| Manual-to-circuit gap | Going from a component datasheet to a working breadboard layout requires expert knowledge. |
| Slow iteration | Making design changes requires re-opening files, re-routing connections, and re-exporting. |

# 3. Solution Architecture

CircuitMind is built as a web app with three integrated layers:

## 3.1 AI Agent Layer

Powered by the Claude API (claude-sonnet-4-20250514). The agent handles two core tasks:

- Natural language → structured circuit JSON (parsing intent from user messages or uploaded PDFs)

- Iterative modification — accepting follow-up instructions like “add a 10kΩ resistor between pins 3 and 5” and updating the circuit definition accordingly

## 3.2 Backend Layer

Python (FastAPI) server responsible for:

- PDF ingestion via PyMuPDF — extracts component specs and pin diagrams from uploaded datasheets

- Prompt orchestration — maintains conversation history and injects circuit state into each Claude API call

- Circuit validation — sanity checks the agent-generated JSON before sending to frontend

## 3.3 Frontend Layer

React app with two main panels:

- Chat panel — conversational interface with the AI agent, showing the iterative design dialogue

- Canvas panel — interactive breadboard visualization using Konva.js, with animated current flow

# 4. Circuit JSON Schema

This is the central contract between the AI agent and the frontend renderer. All three sub-teams must align on this before writing any code.

{

"components": [

{ "id": "R1", "type": "resistor", "value": "10kΩ", "position": [3, 5] },

{ "id": "LED1", "type": "led", "color": "red", "position": [5, 5] },

{ "id": "BTN1", "type": "button", "position": [7, 5] }

],

"connections": [

{ "from": "VCC", "to": "R1.pin1" },

{ "from": "R1.pin2", "to": "LED1.anode" },

{ "from": "LED1.cathode", "to": "BTN1.pin1" },

{ "from": "BTN1.pin2", "to": "GND" }

],

"power": { "voltage": 5, "source": "VCC" },

"metadata": { "name": "LED blink circuit", "description": "Basic LED with button trigger" }

}

Supported component types (MVP scope):

- resistor, led, capacitor, button, wire, power_rail

Each component has a grid position [col, row] that maps to a breadboard hole location.

# 5. Team Structure & Responsibilities

| Person | Role | Owns | Key Output |
| --- | --- | --- | --- |
| P1 | AI Agent | Claude API prompt engineering, conversation state, JSON output | Working agent that outputs valid circuit JSON |
| P2 | Backend | FastAPI server, PDF ingestion (PyMuPDF), API routing | REST endpoints: /chat, /upload-pdf, /validate-circuit |
| P3 | Frontend | React app, chat UI, Konva.js canvas, component rendering | Breadboard canvas that renders circuit JSON |
| P4 | Circuit/Viz | JSON → breadboard layout mapper, current flow animation | Animated current flow on the breadboard canvas |

P1 and P2 must align on the circuit JSON schema and API contract by Hour 2. P3 and P4 can work against a static mock JSON until the agent is ready.

# 6. 12-Hour Build Timeline

| Time | Milestone |
| --- | --- |
| Hours 0–2 | - Lock circuit JSON schema - Set up repo, environment, and API keys - Skeleton frontend (chat + empty canvas) - FastAPI server running with /health endpoint |
| Hours 2–5 | - P1: Agent prompt working — outputs valid JSON from natural language - P2: PDF → text pipeline done - P3: Chat UI functional, static mock circuit renders on canvas - P4: JSON → breadboard layout mapper complete |
| Hours 5–8 | - Full end-to-end loop working (even if rough): type a circuit → see it on the breadboard - PDF upload → circuit generation working - Basic current flow animation on canvas |
| Hours 8–10 | - Chat-to-modify loop working (change one component per turn) - Current flow animation polished - UI cleanup: layout, colors, loading states |
| Hours 10–12 | - Demo rehearsal and bug fixes (critical path only) - Record a backup demo video - Prepare pitch: problem, demo, tech stack slide |

# 7. API Contracts

## POST /upload-pdf

Request: multipart form with PDF file. Response:

{ "extracted_text": "...", "components_hint": ["NE555", "10kΩ resistor", ...] }

## POST /chat

Request:

{ "message": "add a red LED between pins 5 and 7",

"conversation_history": [...],

"current_circuit": { ...circuit JSON... } }

Response:

{ "reply": "I added a red LED at position [5,7] and connected it to R1.",

"updated_circuit": { ...circuit JSON... },

"changes": ["added LED1", "added connection R1.pin2 → LED1.anode"] }

# 8. Frontend Component Map

- App.jsx — root layout, splits chat panel (40%) and canvas panel (60%)

- ChatPanel.jsx — message history, input box, sends POST /chat

- BreadboardCanvas.jsx — Konva.js Stage with layers for rails, holes, components, wires

- ComponentRenderer.jsx — maps component type → visual shape (resistor = rectangle, LED = circle, etc.)

- CurrentFlowAnimation.jsx — animates dots along connection paths using Konva Tween

- PDFUpload.jsx — drag-and-drop PDF upload, calls POST /upload-pdf, seeds first chat message

# 9. Explicitly Out of Scope (MVP)

The following are cut to protect the 12-hour timeline. They can be mentioned as future work in the pitch.

- SPICE simulation — current flow is animated visually, not electrically calculated

- 3D CAD / SolidWorks-style modeling

- Real code execution sandbox (show generated code, don’t run it)

- More than 6 component types

- Multi-user collaboration

- Saving / exporting circuit files

# 10. Demo Script

The judges see this 90-second flow:

- Open the app. Empty breadboard is visible on the right.

- Type: "Build me a basic LED circuit with a button."

- Circuit appears on breadboard. Current flow animates from VCC to GND.

- Type: "Change the resistor to 4.7kΩ and make the LED blue."

- Canvas updates live. Agent explains what changed.

- Upload an Arduino Nano datasheet PDF.

- Agent reads it and suggests a circuit using pins from the datasheet.

Make sure steps 1–4 are flawless. Steps 5–6 are a bonus.

# 11. Tech Stack Summary

| Layer | Technology |
| --- | --- |
| AI Agent | Claude API — claude-sonnet-4-20250514 |
| Backend | Python 3.11, FastAPI, PyMuPDF |
| Frontend | React 18, Konva.js, Tailwind CSS |
| Dev / Infra | Vite, uvicorn, Docker (optional) |
| Source Control | GitHub — monorepo: /frontend, /backend |

CircuitMind • Hackathon 2026 • Lock the JSON schema first, ship everything else second.